2020-02-14 PFRC-2 Run Summary

prepared 2020-02-(14-17) S.A. Cohen, et al.

RMFo @ 4.3 MHz, air-gapped, two-turn antennas, 60” RG-226 transmission lines, ground plane between antennas and Lexan vessel. RMF power system: SRI->AR100LM->8KD-> 200kW-> hybrid splitter-> tanks. “Safety” on RMF (pulse length x power -> 1% duty factor). BB power supply and 2 Magnapower. BN-covered HTS-FCs, installed Sept 2011.

Goals: Understanding DLs

Setup:

a) SRS: max 0.95 Volts.

b) Estimate fv ~ 4.3xx MHz

c) Pr/Pf ~ 1/4 -1%; max Pf ~ 25-55 kW; max Pa ~ 36% Pf

d) FM: no

e) Duration: 3-8 ms;     time between pulses: 1.013 s

f) FCs: room temp & LN2

g) Tank circuits -> 2 antennas

h) No antenna Rogowskis:

i) Rotation direction: 90° throughout discharge

j) cc Slow baratron: 0.56-0.74 mT

k) Base vacuum: 4e-7 T FEC gauge;

l) PSEC ~ 1.8 mT

m) BB:  I = 165-385 A;  MagnaPower: I = 181 A

n) Helmholtz coils:  (4+4)+4  and  4+ (4+4)

o) Nozzle: 106 A

p) Helicon: 17 W net seed plasma

q) -wave: ~ 1.9 MHz LPF, nmax ~1.8e12/cc,

r) monochromator

s) max ?? Vs – perhaps 7 Vs!!

Experiments:  DL signals: DL1 vs external and DL1 vs 6; with and without BB; vs pulse length; with and wo LN2.

| Experiment 1 – Internal (1) vs external DL (3ms) IL-2=180A, Pf ~ 47 kW, Pa~13kW, p~ 0.69mT, DL cases grounded; ne~0.9e12/cc; yellow = external; olive = DL1; brown = DL6 Noisy ne(t). Flat <ne> 10 :36 :24 | Observations: 1) DL1 shows ~ linear increase during RMF. Charles speculates capacitive coupling. Why not real? 2) All 3DLs show + signal (overshoot) after pulse. If this is flux returning through FCs then BIG flux, > 5 Vs. 3) BB noise (3-phase => 360 Hz) seen in unintegrated DL. 4) DL6 shows the same sharp rise/fall interpreted as diamagnetic signal in DL1 --How possible so far from z=0? |
| --- | --- |

| Experiment 2 – Higher B; IBB=382A; IMP = 180A (3ms) | Observations: 10:43:16 1) DL1 rose to 5 Vs and no post-shot overshoot! 2) External and DL6 unchanged 3) Better Pa balance & higher Pa than lower BB 4) Same post-shot DL1 behavior to t = 45ms 5) note difft DL1 slopes, during vs post shot 10:45:32 6) Grounded DL Pomona boxes – no change (not shown) |
| --- | --- |

| Experiment 3 – Lower B; IBB=0A; IMP = 180A (3ms) | Observations: 10:53:14 1) DL1 rose to 7 Vs and no post-shot overshoot! This is BEFORE LN2. 2) External DL has small over-shoot; DL6 overshoot unchanged 3) Poor Pa balance 4) note diff’t DL1 slopes during vs post shot 5) diff’t ne (t), periodic MHD after t ~1.5 ms. 10:51:03 |
| --- | --- |

| Experiment 4 – With LN2; IBB=0A; IMP = 182A (3ms) | Observations: 10:53:14 1) For all DLs lower post-shot signal/overshoot! 2) Lower post shot External DL-less post shot overshoot; DL6 post-shot unchanged 3) Poor Pa balance 4) Note, as before, difft DL1 slopes, during vs post shot 5) diff’t ne (t), periodic MHD after t ~1.5 ms. 10:51:03 |
| --- | --- |

| Experiment 5 – With LN2; IBB=0A; IMP = 182A (8ms) | Observations: 11:35:32 1) DL1 signal still grows linear with time. 11:34:32 2) All DLs post-shot – no overshoot 3) Poor Pa balance12:09:16 4) Pa noisy and high after 5 ms - 16kW/55kW 5) 3-plateau ne(t) 12:09:13 6) Periodic MHD t=2-5 ms. 10:51:03 7) Note, as before, difft DL1 slopes, during vs post shot 8) Note diff’t DL behaviors a plateau changes. DL6 and DLext smaller; DL1 bigger |
| --- | --- |
| Experiment 6 – With LN2; IBB=210A; IMP = 182A (8ms) | Observations: t >12:12 1) Very noisy Pa To 20kW/55kW 2) Pr rises at ~4 ms 3) Smaller DL signals than at lower B 4) DL6 does not increase as steeply for t> 4 ms. 5) Density falls a lot after t~2.5 ms Soon after ARCING South cable, inside box, in fiting |

Conclusions/recommendations/questions 1. Can get no DL overshoot both with and without LN2. Cause of overshoot still a mystery. 2. Don’t know if sharp DL rise and fall at pulse’s beginning & end are real!! 3. Don’t know if linear DL rise during pulse is real. 4. Why DL6 signal at all? 5. RC of cable plus scope? 6. Operate Magnapower in constant current mode 7. Gas puff 8. Test additional DLs, even ones not encircling plasma 9. Model DL signal with PSpice, include capacitive coupling 10. Take notes on grounding Pomona boxes. 10. Meausre DL temps during run (quickly .