11/21/2022 PFRC-2 Run Summary

prepared 11/21/22 S.A. Cohen

RMFo @ 1.803-4 MHz, air-gapped, two-turn antennas, ground plane between antennas and Lexan vessel. RMF power system: SRI -> AR100LM -> 8KD -> 200kW -> quad splitter -> RG-217 transmission -> tanks -> antennas. BB and Magnapower for L-2 coils.  Magnapower for nozzle coil. BN-covered HTS-FCs.

Goal: RMF: measure tD with Argon

Setup:

- SRS: to 1.7 Volts (80 kW).

- Faraday ground planes ungrounded then grounded

c) Base vacuum: 5.1 e-7 T FEC;  RGA H2O peak ~ 40x N2 peak & ~ 5x H2 peak

c) Ops at: Po ~ 0.50 mT CC Ar

d) BB I=200-404 A and MP I =200-400 A

e) Nozzle coil: 50 and 350 A

f) L-2:  4 x 8 + 8 x 4

g) Helicon: 0.1 V, ~70 W

h) Antenna I’s balanced, ±1%; P’s a bit different, Pfs & Pas at ±6%. Phase 93°±3°

i) 4.5-ms pulses, 1/s rep rate. (A few at 8 ms)

j) Vcap bank = 16 kV

k) RMF amp filament V, I:  14.79->15.24 Volts and 202.4->206.3 A

Results:

- BBF with Ar even at relatively low power, 31 kW, If B low, ~ 140 G.

- RGA 72% Ar, 23% H2, 3% H2O, 1% CO

- ne with Ar to ~1e13/cc, ne decay time ~ 1/3 ms

- In Pearsons, 3 w at least 30 dB below w with Ar.

- Vc in helicon -1 kV, first falls then rises during RMF.

- RMF antenna real Phase ~ 90° on Pearsons.

- With Ar:  Pa ~ 12 kW at Pf ~31 kW (39%) and Pa ~ 20 kW at Pf ~ 56 kW (36%).

- Tuning RMF.  f = 1.803-1.804 MHz

- Slow then fast ne rise. Slow ~200 ms; fast ~ 5 ms. ne noisy, ±30% @ 1-5 kHz

- At higher B (IL-2 ~ 400 A), 4 kHz osc in Pa.

- Spreadsheet parameters ~ consistent with experiment. Te = 100 eV, Emax,e (Hamiltonian) = 100 eV,  Ti = 0.2 eV, amu = 40, rs ~ 5 cm, B(0,0) = 200 G, fRMF ~ 1.6 MHz, Pa = 20 kW, tE ~ 10 ms, if approx. Ar radiation losses included.)

- Some X-rays, likely from discharge initiation.  ~ 9 eV/pixel

Recommendations/questions - Raise power above 90 kW. Non-linear field penetration threshold? Harmonics? - Bias Ta plate and cup + and -. Measure I’s. - Get X-ray data vs t to see if/when fast/energetic e- created - Get spectrometer data to “see” Te. - Re-ground Ground planes. - Ar runs to measure tD: Vary Inozzle, IL-2, fill pressure, Pf. - Transition Ar -> H2 runs: increase H2 partial pressure.