12/21/2022 PFRC-2 Run Summary

prepared 12/22-25/22 S.A. Cohen

RMFo @ 1.800 MHz, air-gapped, two-turn antennas, ground plane between antennas and Lexan vessel. RMF !ower system: SRs -> AR100LM -> Henry 8KD -> 200kW -> quad splitter -> RG-217 transmission -> tanks -> antennas. BB and Magnapower for L-2 coils.  Magnapower for nozzle coil. BN-covered HTS-FCs.

Goal: X-rays as f(RMF power), no BD to BD; mostly with H2 fill gas.

Setup:

a) SRSRMF to 1.67 Volts.  fPRMF to 82 kW; f = 1.8013, 1.8008 MHz

b) 6-ms-duration RMF, 1/s, phase (low power)85-(high power)95°

c) SRShelicon: 0.28 Volts.  heliconPmax ~ 650 W

b) Base vacuum: 3.3 e-7 T FEC

c) Ops (most) at: Po ~ 0.32 CC H2. No BD. Later add Ar, get BD at 82 kW

d) RGA says H2 can exceed 99%

f) IL-2 ~ 250, 227 and 150,150 A, and In =100 A.

g) Ratio of antenna currents (Pearson) TB/NS ~ 0.82

Results:

| - No BDs with pure H2 at Pf to 61 kW. - When no BD, Dnmax first increases with Pf (to 13 kW) then decreases. This is despite the flashes getting brighter. Why? How? the absorbed power increases! Perhaps Te rises but ne falls, the rich get richer? Pickup? - Good balance (±2%) but poor phase (83°) in antenna currents at low power. Opposite at high power. - Concerning Pa: + at RMF beginning and – at end. DE ~ 0.2 J/antenna pair. - At higher power, osc in Pa at beginning of RMF. - Pr/Pf ~ 1% N/S and 2% T/B - ne first rises fast (50 ms), then plateau, then slow (500 ms). - At end of RMF, ne falls, (t ~ 40 ms), then briefly plateaus (50 ms), then falls more slowly (t =200 ms). | Pa at start 0.1 ms/div. Lower power No osc 2 ms/div . ne(t) 10 mR/div ne rise. 0.2 ms/div ne fall 0.2 ms/div |
| --- | --- |

- <Vc> from helicon ~ -2.7 kV. Little change ( + 50 V) if no BD. With BD,  <Vc> ~ -2.4 kV and becomes even less negative (-2 kV) for 0.4 ms after RMF

- When BD – this also means higher power- CRAZY X-ray spectra, esp in CC and nozzle SDDs – Noise!! PICK UP!! Better Sheilding needed. Examples of craziness.

Pa ~ 5 kW, 5.6 ms Nozzle SDD                    Midpoint SDD                Nozzle SDD

- ~600 Hz osc seen in Vc and <Vc> during RMF. Had previously see 600 Hz osc in interferometer at high pressure BUT after the RMF.

- Density osc at 6, 12, 18, 24, & 30 kHz, decreasing 5 dB as f increases. Also at 700 kHz. Small one at 633 kHz

- Back to trying to understand Pa oscillations at start-up.

0.02 ms/div,  Pf 0.02 ms/div,  Pf nd Pr0.05 ms/div,  Pf nd Pr

Osc’s not seen in constancy of Pf

Recommendations/questions - Understand after-RMF “Noise (osc)” seen in ne at high pressures. - FFTs of ne when osc - Learn how to avoid/stabilize osc. - Why does Dne fall with increasing RMF power when no BD? Pickup? Artifact?