12/22/2022 PFRC-2 Run Summary

prepared 12/22-25/22 S.A. Cohen

RMFo @ 1.800 MHz, air-gapped, two-turn antennas, ground plane between antennas and Lexan vessel. RMF power system: SRS -> AR100LM -> Henry 8KD -> 200kW -> quad splitter -> RG-217 transmission lines -> tanks -> antennas. BB and Magnapower for L-2 coils.  Magnapower for nozzle coil. BN-covered HTS-FCs.

Goal: X-rays as f(RMF power, location), no-BD to yes-BD; start with pure H2 fill gas, then add He, Ar. Get BD only with added Ar.

Setup:

a) SRSRMF to 1.9 Volts.  fPRMF to 94 kW; f = 1.800-1.8028 MHz

b) 6-ms-duration RMF, 1/s, phase (low power)85-(high power)95°

c) SRShelicon: 0.28 Volts.  P ~ 650 W

b) Base vacuum: 3.3 e-7 T FEC

c) Ops at: Po ~ 0.27 CC H2. (Later add ~0.2 mT Ar because no success with H2 or added He)

d) RGA says H2 can exceed 99%

f) Mostly IL-2 ~ 130,150 A, and In =100 A.

g) Ratio of antenna currents (Pearson) TB/NS ~ 0.82

Results:

- No BD with pure H2 to 84 kW at IL-2 ~ 200 A and PH2 ~ 0.23 mT

- With BD (from adding 0.20 mT Ar  to 0.27 mT H2): <Vc> from helicon ~ -2.7 kV. Little change (+25 V) if no BD. With BD,  <Vc> ~ -2.4 kV. See later item, #24.

For BD needed to lower B and raise Pf. Initially  tD = 0.7-2.0 ms.

- BD data mostly at IL-2 = 130 A MP and 153 A BB. Pf = 89 kW.

- Pr/Pf for T/B(1/4%) show 4x less reflected power than N/S(2%). OPPOSITE of earlier in day (with no BD), even with tuning.

- If Ar partial pressure lowered to 0.18 mT, lost BD.

- With BD, Radial SDD shows large low E signal (below channel 38, E ~ 340 eV), 20X bigger than O peak. Large input rate.

- With BD, Nozzle SDD shows small low E signal, ¼ of O peak. Low input rate

- With BD, CC SDD shows REALLY HUGE low E signal (below channel 50, E ~ 450 eV), 150x O peak. HUGE input rate.  WOW!!! Nozzle signal @ 1 keV > CC, Radial

Informative

Exciting

Very

exciting

Radial SDD                             Nozzle SDD                          CC SDD

1 ms/div CC SDD

(Above) Aperture 4; With Aperture 0 (not shown), no X-rays seen by CC SDD

Radial SDD; LOS 1

Time behavior needs to be examined. Are these low or high energy x-rays? Is the drop in brightness seen by the Radial due to reduced neutral density or changed profile or ..??

20.ne,max ~ 8e12/cc (for 8 cm) ffund ~ 7.5 kHz. After discharge ne falls 1/e in t ~ 0.1 ms.

1-ms/div, 2-Rad/div. Data (oscillation size) imply rplasma ~ 4.5 cm

21.<Vc> starts at -2.7 kV. The <Vc> increase above -2.7 kV increases with Pf, f ~ 7.5 kHz.  Note, Vc goes close to 0 for Pf > 88 kW, while <Vc> stays negative.

22. With added Ar and no BD: FFT of Vc  21 kHz, 42 kHz (-20 dB)

23. With added Ar and BD: FFT of Vc7 kHz, 14 kHz (-5 dB), D<Vc> = +900 V

0

0

0

24.Raising Pf increases the increase in <Vc> during RMF.

RMF on     BD                                                  RMF off

Pf = 87 kW; FFT: 10 kHz/div Pf = 89 kW Pf = 94 kW; FFT: 5 dB/div

1 ms/div: Vc(t) (top trace): 2 kV/div.  <Vc(t)> (bottom trace):  500 V/div, Average of ~ 300 discharges.  f harmonic disappears at higher power. tBD shorter at higher power.

<Vc> increasing with time  Try LONGER discharges.

Recommendations/questions IMPORTANT DAY with BD – saw effects of Pf on Vc, saw low-energy x-rays diff’t at diff’t positions. CC, Radial, and Nozzle SDDs - More FFTs of Vc and of ne vs power. Vc is likely important; it may be an indicator of ne lost from plasma core (z = 0) and of the “real” electron temp in an FRC’s SOL. Why does Vc gets to 0 at high Pf?? Understand Vc and <Vc>. - Learn how to avoid/stabilize osc. Apply electric potentials: Ta plate, DL, FCs, C cup. Gas puff. Symmetry? RMF Phase & balance? Shape of B?….. - STUDY SDD spectra. Diff’t locations (z,r)!!! Resolve real from artifact. - Radial vs CC SDDs – compare radial profiles spectra and brightness vs time - Suspicion/speculation: X-ray spectra – the high energy (> 500 eV) x-rays are from the helicon (edge plasma) the BRIGHT low energy (< 400 eV) X-rays are from an FRC heated by RMF heating & producing truncated distributions!! - Make longer pulses, to see if <Vc> continues to climb. Effects of B, of Pcc. - Use LN2. Mylar??!!?? - Higher power RMF, to 120 kW & higher. - Extract ne from low-E X-ray spectra!!! - Probes!!! - Gas puff to see if X-rays get brighter. Experiments to do with SDDs when BD - With CC SDD - See if low-E, <400 eV, X-ray signal is real: - Aperture sizes: Compare high- with low-E signals sizes vs aperture size - % ST: (0.1, 0.3, 0.5, 1, 2, 3, 4 %). Look for PPU. - Vary peaking time: 1, 2, 5.6, 10, 25.6 ms. - Effect of mylar - Understand “location” and source/cause of (low amplitude) high-E, above ~ 500 eV, X-ray signal. - Pulse helicon power during RMF (helicon SRS burst mode) If RMF duration is 5 ms – then helicon on 1 ms, off 1 ms, on 1 ms, off 1 ms, then left on awaiting the next RMF pulse. Accumulate SDD data for the middle 0.8 ms of each 1 ms section. See if high E signal disappears when helicon is off. - With Radial and Nozzle SDDs - Radial scans to see spectra vs r - Compare Nozzle SDD to Radial SDD data, esp at low energy. - Use the O peak as the “normalizer.”