2023-06-27 PFRC-2 Run Summary

prepared 2023-07-02 S.A. Cohen

RMFo @ 1.8 MHz, air-gapped, two-turn antennas, 28” RG-217 transmission lines, ground plane between antennas and Lexan vessel. RMF power system: SRI->AR100LM+ENI->>8KD-> 200 kW. BN-covered HTS-FCs.

Goals:  Forward (co) and Reverse (cntr) RMF rotation direction

Setup:

a) SRS: 1.34-1.78 Volts

b) IL-2: 150, 300 A (110 G, 216 G)

c) Pr/Pf ~ 1% ;

d) Pf ~ 27-72 kW

e) RMF: 1.798 MHz

f) Duration: 6 MHz ms.

g) Base vacuum: 3.4e-7 T FEC

h) Ops at: 0.4-0.82 mT, MC Ar: 0.64, 0.85 and 1.1 mT.

i) Nozzle: 100 A

j) Helicon: 120 W seed plasma

Results:

- About 3000 plasma pulses.

- Absorbed power in usual rotation direction (co), 25% of Pf

- maxPf = 16, 30 kW, maxPa ~ 5.5 kW during flat top.

4.  Phase match mediocre: 82-85° for co (goal 94°), -98° for counter (goal is -86°)

5.  Minimum power for Breakdown at various pcc and B:

Direction     IL-2(A)          Pf(kW)    tD (ms)   ts (ms)  tf (ms)    pcc (mT)  tn(1e11/cc)

Co                150              12           1             0.2        0.02        0.85           2.6±0.5

Co                150              14           1.2           0.2       0.02        1.107.        2.6±0.5

Co                150              15.5        0.85         0.2       0.02        0.64           2.6±0.5

Co                300              30           2.4           1.4       0.02        0.63           4±0.5

Co                300              30           2.6           0.2       0.02        0.85

Co                300              30           2.0           0.2       0.02        1.10

Cntr                300               45                                                      1.10 (no BKDN)

Cntr                300               60                                                      1.10 (no BKDN)

Cntr                300               72         0.36           0.1       0.1          1.10           5.4±0.5

For low power cntr, <50 kW,  n falls ~ 4e11/cc

At Pf ~ 60 kW, n finally rises ~ 4e11/cc

P = 22 kW                                                      P = 60 kW

6.At Pf = 72 kW – BBF. Density = 5 radians = 6.7 e12/cc. NOISY n!!     tn ~5.4e11/cc

Major decrease in “noise” in ne at 1.5 ms. Perhaps in Pa also.

Pa/Pf ~ 25%

First time BRKDN with CNTR RMF rotation