2023-06-28 PFRC-2 Run Summary

prepared 2023-07-03 S.A. Cohen

RMFo @ 1.8 MHz, air-gapped, two-turn antennas. Ground plane between antennas and Lexan vessel. RMF power system: SRI>AR100LM+2nd RF amp->>8KD-->> 200 kW. BN-covered HTS-FCs.

Goals:  Reverse (cntr) RMF rotation direction

Setup:

a) SRS: ->2.1 Volts

b) IL-2: ~ 250A (180 G) (“one shot” at B ~ 230 G)

c) Pr/Pf ~ 3 % when BBF;

d) Pf  to 105 kW

e) RMF: 1.7988 MHz

f) Duration: 6 MHz ms.

g) Base vacuum: 3.4e-7 T FEC

h) Ops at: 0.54 mT, Ar

i) Nozzle: 100 A

j) Helicon: 320 W seed plasma

Results:

- About 5000 plasma pulses.

- When no BKDN major imbalance of Pa N/S vs T/B (Pf ~ 72 kW) Green is T/B

- Absorbed power in CNTR rotation direction, 28% of Pf in antenna whose power is being absorbed.

- Density rises to 0.94e12/cc and flattops when NO BBF. Contrast that with tn~ 2e11/cc.

- With no BBF.  Density rise ~ 0.25 ms.       tD ~ 0.3 ms.     Density decay ~ 0.4 ms. Broad FFT (not shown)

- First BBF at maxPf ~100 kW. maxPa ~ 26 kW. Pa balanced N/S and T/B

- Very noisy high ne~1.5e13/cc, maybe to 1.9e13/cc.  tD ~ 0.25 ms (maybe a bit shorter).      Note ne initially falls a bit before tD , i.e., from t = 0 to 0.2 ms.    Pf ~ 100 kW.

- Phase match mediocre:  -98° for counter (goal is -86°)

- Note no “poorer coupling” in Pa before tD as common – the rule – for Co rotation,

- SRS      Vav      P(kW)

1.9        .457        90

2.0.       .482      100

2.1.       .497.     106

Tried to find threshold by slowly raising SRS: No BBF for SRS = 1.90, 1.92, 1.94, 1.96

THOUGH, the density increases and a ne peak (5e12/cc) appears near the beginning.  2 V/div, 1 ms/div

BBF finally at SRS = 1.98 -> Pf ~ 97 kW.

- Able to get BBF at IL-2 BB= 375 and Imagnapower = 250 A.  B(0,0) ~ 230 G