2023-08-01 PFRC-2 Run Summary

prepared 2023-08-17,18 S.A. Cohen

RMFo @ 1.8 MHz, air-gapped, two-turn antennas. Ground plane between antennas and Lexan vessel. RMF power system: SRI>AR100LM+2nd RF amp->>8KD-->> 200 kW. BN-covered HTS-FCs.

Goals:  Testing RMF amp; RF reflection (resonant frequency), with and without H2 plasma, during slow and fast density-rise phases; measure Pr, Dfresonance

Setup:

One MP PS for L-2 coils died.

a) IL-2: ~ 220, 320 A (BB only); (MP problems)

b) Nozzle: 100 A

c) RMF: 1.7978 MHz

d) Cap bank: 17 kV

e) SRS: ->1.51 Volts

f) Pf to 100 kW; Pa to 30 kW

g) Pr/Pf ~ ½ %

i) Duration: 3 ms.

i) Ops at Pcc ~ 0.6 and 1.5 mT, H2

k) Helicon: 300, 500 W

Results:

- About 6000 plasma pulses.

- ne max ~ 4 Volts -> 5e12/cc

pcc = 0.6 mT, 50% ne osc 14 then 11 kHz fundamental; ne flat with t.

pcc = 1.5 mT,    ne rises with t.

- When PRMF first applied, density pump out (in “slow density rise” phase) at low pcc.

- But not at high pcc.

- pcc = 0.6 mT tD = 0.2 to 10 ms f(Pf).      Density decay first @ 0.07 ms then longer.

- Good antenna current balance. Good tuning match N/S to T/B. Pf vs time

- Pr/Pf < 1%

- Phase match, 99° to 106° (should be 94°)

- Often imbalance in Pa. Pa(N/S) ~ 1.4-2	 x Pa(T/B)

- Pa before BBF 40-70% Pa during BBF

- Resonant frequency. Three cases: 1) no plasma; 2) during slow density rise phase; 3) in steady state t~ 1.5 ms

- Pa ~ 30 kW, Pf ~ 100 kW; pcc ~ 1.5 mT This shift of the resonant frequency corresponds to 4 cm flux exclusion.

- At resonance, more ne oscillations